Reactive Display System: Merging Sensor Inputs with Visual Output Interfacing DHT11 Humidity/Temperature sensor using STM32 Microcontroller & an LCD display with I2c.

EE5203 MICROPROCESSORS

COURSE PROJECT

Ahmat Saleh Bachar

1900004976

Awab Mohamedkhair

2300003076

Lecturer Basri ERDOĞAN

January 2024

- ABSTRACT

This project focuses on the integration of the DHT11 sensor with the STM32 Microcontroller. The objective is to create a robust system that accurately monitors and displays real-time humidity and temperature data. The DHT11 sensor is connected to the STM32 microcontroller, which is the central processing unit. The interfacing process involves configuring the microcontroller, implementing the necessary communication protocols, and developing code for seamless data transfer. The project explores the challenges and solutions encountered during the interfacing process, providing insights into the reliability and precision of the obtained results. The outcomes of this project contribute to the broader understanding of sensor-microcontroller interactions and open avenues for applications in temperature and humidity monitoring systems.

- LIST OF COMPONENTS

- SMT32F401RE

- DHT11 Sensor

- 16x2 LCD DISPLAY

- I2C module (PCF8574)

- JUMPER CABLES

- INTRODUCTION

In this article, we explore the creation of a reactive display system using an array of cutting-edge components. We delve into the workings of the DHT11 sensor, the STM32 microcontroller, the I2C module, and the 16x2 LCD to demonstrate how they seamlessly interact to provide real-time, insightful data.

- Component’s Overview

- STM32F401RE

The STM32 microcontroller is the heart of our reactive display system. It provides the processing power and control necessary to capture data from the DHT11 sensor and communicate with the12C module and 16x2 LCD. With its advanced features and capabilities, the STM32 microcontroller enables us to create a dynamic and interactive display that seamlessly reacts to sensor inputs.

https:// HYPERLINK "https://www.st.com/resource/en/datasheet/stm32f401re.pdf"www.st.com HYPERLINK "https://www.st.com/resource/en/datasheet/stm32f401re.pdf"/resource/ HYPERLINK "https://www.st.com/resource/en/datasheet/stm32f401re.pdf"en HYPERLINK "https://www.st.com/resource/en/datasheet/stm32f401re.pdf"/datasheet/stm32f401re.pdf

- DHT11 Sensor

The DHT11 sensor is a vital component in our reactive display system. It measures temperature and humidity levels, providing valuable data for the display. By integrating it with the STM32 microcontroller and the 12C module, we can capture real-time information and display it on the 16x2 LCD. This allows us to create a visually appealing and informative display that responds to changes in the environment.

https://www.mouser.com/datasheet/2/758/DHT11-Technical-Data-Sheet-Translated-Version-1143054.pdf

- 16x2 LCD display

The 16x2 LCD display is a key component of our reactive display system. It provides a visual interface for users to interact with the system and displays relevant information. With its compact size and ability to display two lines of text with 16 characters each, the 16x2 LCD is ideal for showing real-time data from the sensor inputs and creating an intuitive user experience.

- I2C module

The IC (Inter-Integrated Circuit) module plays a crucial role in our reactive display system. It facilitates communication between the STM32 microcontroller and the 16x2 LCD, allowing them to exchange data and instructions. The I2C module simplifies the wiring and supports multiple devices on the same bus, making it a convenient and efficient solution for our project.

- PROCEDURE

- The Microcontroller’s Configuration

Let us dive into the core component of our project, the STM32F401RE microcontroller serves as the brain of our operation, orchestrating the seamless integration with the DHT11 sensor. Here are some features of this microcontroller: The STM32F401RE microcontroller runs at speeds of up to 84 MHz with its ARM Cortex-M4 core. It's good at handling data with its Floating Point Unit (FPU) and DSP instructions. It has a 512 Kbytes of Flash memory and 96 Kbytes of SRAM as its memory storage, making it great for our needs. This microcontroller can easily communicate with other devices using interfaces like I2C, SPI, I2S, USART, SDIO, and USB 2.0 OTG, connecting smoothly with the DHT11 sensor. It can work in different range of temperatures (-40 to +105 °C) and runs on a power supply of 1.7 V to 3.3 V, which is handy for various applications like controlling motors, industrial systems, and sensors in mobile phones.

- Setting Timer

Setting up a timer is crucial to interface the DHT11 Sensor. The DHT11 sensor requires precise timing for initialization process and data sampling. By configuring a timer, one can ensure accurate and consistent intervals for sampling temperature and humidity data from the sensor. This is particularly important as the DHT11 communicates data in a specifi timing protocol. The HIS(High-Speed Internal oscillator) internal clock source of the microcontroller is set to run at 50MHz.

Figure 1: TIM3 Configuration

- I2C Module

I2C1, the communication module which is attached to the 16x2 LCD display is enabled from pinout configuration under Connectivity.

- Pinout Configuration

Figure 2: STM32F401RE Pinout Configuration

PA0 is configured to be the DHT11 Sensor’s data input.

PB8 is configured as the I2C’s Serial Clock line (SCL), it carries the clock signal that synchronizes data transmission between the master (microcontroller) and the slave devices on the I2C bus.

PB9 is configured as the I2C’s the Serial Data line (SDA), it carries the actual data being transmitted between the master and slave devices on the I2C bus.

- Interfacing the DHT11 with the Microcontroller

The DHT11 digital sensor is an embedded sensor used to measure temperature & humidity in the surroundings and gives calibrated digital output. It can measure temperature in the range of 0°C to 50°C with ±2°C accuracy. Its humidity range is from 20% to 80% with ±5% accuracy. It is a small, low cost and easy-to-interface embedded sensor.

Figure 3: DHT11 Pinout

2.3. Interfacing I2C LCD modules with microcontrollers

is a common requirement in many embedded applications. LCDs provide an easy-to-read visual output that can display various types of information, such as sensor readings, status messages, and user inputs. However, traditional methods of interfacing LCDs with microcontrollers can be complex, requiring many GPIO pins and complex wiring.

This is where I2C comes in. I2C is a communication protocol that allows multiple devices to communicate with each other using just two wires: a clock line (SCL) and a data line (SDA). I2C is a popular choice for interfacing LCD modules with microcontrollers because it requires only two pins, simplifying the wiring and reducing the number of GPIO pins needed

I2CLCD DISPLAY

The Hitachi HD44780 is a popular LCD controller used in many 16×2 character LCD displays. This controller is widely used due to its low power consumption, ease of use, and reliability. The HD44780 supports up to 32 characters (16 columns x 2 rows) and offers various features such as character generation, display data RAM, and display on/off control.

Figure 4: I2CLCD Display

Features of the 16×2 LCD Display

*Low power consumption

*16×2 character display (16 columns x 2 rows)

*Supports up to 32 characters

*Supports alphanumeric characters and symbols

*Built-in character RAM for up to 8 user-defined characters

*8-bit parallel interface or 4-bit parallel interface

16×2 LCD Pinout and Pin Description

Figure 5: LCD Pinout Description

PCF8574 I/O Expender

for expanding the number of I/O pins available to a microcontroller.

The PCF8574 is an 8-bit I/O expander that communicates with microcontrollers through the I2C protocol. It is commonly used to expand the number of GPIO pins available to the microcontroller. The PCF8574 can be used to interface with various peripherals, including LCD displays, LEDs, and sensors. It has eight I/O pins that can be used for both input and output operations. The PCF8574 is also designed to work at low voltage levels, making it suitable for battery-powered applications. Additionally, multiple PCF8574 devices can be connected together, providing an easy way to expand the number of I/O pins even further. Overall, the PCF8574 is a versatile and cost-effective solution